SCENARIO 3 REPORT-GROUP 8

Machine Characterisation

3D Printer

3D printing is a process of building components using layer-by-layer reconstruction. Each of the layers constructed can be seen as a thinly sliced cross-section of the object built [4]. There are 3 main types of Bioprinting, Inkjet Bioprinting, Extrusion-base Bioprinting and Laser-assisted Bioprinting. However, in this particular project, we are focusing on the Extrusion-base bioprinting as it has the most detailed outcome of all. There are 3 main stages executed during extrusion-base bioprinting. Firstly, the pre-processing stage, involves the planning of the model on a computer program as well as setting the 3d printer itself. Following, the bio-printing building phase consists of the material used, being loaded into the printing chamber and pushed into a nozzle, producing a continuous consistent filament and forming the desired assembly uniformly. Finally, the post-bioprinting phase requires the finishing of construction of the structures and removal of any additional supports if used. A gap of 30 minutes is required once the printing is done, to avoid any inhalation of harmful nanoparticles produced during the process. [5]

The 3D printers available in the lab for the manufacturing of the peristaltic pump, are classified as the Ultimaker 3 and Ultimaker 3 extended. The operating software used is called Cura, allowing the visualization of the prototype before being sent out for printing and checking printing parameters. The printing material chosen will be Polylactic Acid (PLA) due to its high resolution and low shrinkage factor, allowing prints having lower alternations/deviations compared to the desired designed components. Additionally, PLA material lowers the printing time as it doesn’t require any build plate heating, resulting in better adhesion.  Some design considerations that should be taken into account, include wall thickness of design, circular objects and design dimensions.

Thin components should be carefully designed, as a wall thickness less than that of the width of the printing line ( 0.2mm) could eventually result in a very weak designed object. To tackle this issue, the walls of the desired objects should be designed to have at least twice the width of the printing line. However, our peristatic pump design does not experience this constrain as all walls are more than twice the width of the print line, as components are design to withstand high water pressure and rotational movement applied during the experiment. Furthermore, an additional constrain of the printer involves circular components being printed out inaccurately, resulting in a greater tolerance and lower precision. Thus, a tolerance analysis should indicate any additional changes required of shapes and sizes to avoid such complications to the design. Finally, the time constrain issue can be faced by increasing the speed for components with no great details and ensuring there is no waste of material by designing any inefficient parts of no use. [6]

Laser cutter is another piece of machinery used in the assemble of the peristaltic pump of this project. The particular piece of equipment involves laser beams to cut the outline of various materials with high accuracy. The process can start by developing one’s idea and using a 2D software, sent the design to the laser cutter.  The pieces produced are clean and highly detailed and ready to use.

The laser cutter available in the lab for this project is the 60W-CO2 by Universal Laser System Inc. Adobe Illustrator is used to send the 2D designs for cutting, engraving and marking them. This is a very fast method, of producing results of consistent tolerances and precisions within a few minutes. The maximum cut depth of the material available (acrylic) is 5.7 mm. Unlike the 3D printer, laser cutting creates holes of bigger size compared to the desired design and shortens the lengths of the sides of the object. This is due to the defocusing of the laser beam, creating larger cuts. Applying this to out object, there were no markable changes noticed as the produced shape was of the right dimensions and outlines. To that end, different parts have to be correctly aligned and calculated, and using minimised height of each layer produced. [7]

Tolerance Analysis

As mentioned above, both of the designing equipment used are not able to design the desired components/assemblies to the right size. This is due to their tolerance. Tolerance can be defined as “the total amount a specific dimension is permitted to vary”. To avoid any inaccurate results produced and ensure the perfect fitting of our components to the final assembly, tolerance analysis is performed on PLA 3D printing and Acrylic Laser cutting test samples – 4 manufactured by each piece of machinery. Firstly, each sample’s dimensional parts were labelled to allow comparison and identification of the physical pieces. The 4 shapes of each manufacturing process were composed of different shapes: cylinders, circular holes, hexagonal holes, extrusions and T-shape openings. Following, 4 consecutive measurements of each shape on the 4 samples were taken and noted down. This allowed for a mean value (µ) of measurements to be calculated, followed by the differences of the mean and true values. Using that information, the standard deviation (σ) and percentage errors were calculated and a Gaussian Graph was plotted using the normal distribution formula:

y=1σ2π e-12x-μσ2

Table 1: Measurements and Calculations for 3D printed samples

Table 2: Measurements and Calculations for Laser cut samples

Using a MATLAB, the graphs were uniformly plotted and analysed for both machines.

Tables 3-7 : Gaussian distribution deviations for 3D printer

|  | Cylinders | Circular Holes | Double Holes | Hexagonal Holes | T-shapes |
| --- | --- | --- | --- | --- | --- |
| Max | 0.74 | 0.66 | 1.52 | 0.45 | 14.23 |
| Min | -0.50 | -0.59 | -2.20 | -0.67 | -14.51 |

Table 11:  ± 3σ confidence intervals (in millimetres) in 2dp

Tables 8-10: Gaussian distribution deviations for Laser Cut

|  | Circular Holes | Hexagonal Holes | T-Shapes |
| --- | --- | --- | --- |
| Max | 5.24 | 0.10 | 0.71 |
| Min | -6.66 | -0.12 | -0.86 |

Table 12:  ± 3σ confidence intervals (in millimetres) in 2dp

Analysing the key statistical information obtained above, the tolerance of the 3D printer is much greater than that of the laser cutter. Furthermore, the measured dimensions of the 3D printer appear to be smaller than those of the true values, whereas the plotted dimensions of the laser cutter appear to be larger and deviate positively. Therefore, it can be concluded that complex, detailed components of the peristaltic pump assembly would be more suitable if 3D printed instead of being laser cut. As lots of holes and fittings appear in the design, 3D printing will be more suitable for accurate connection of components by screws and joined edges.

Some possible sources of error include human error due to lack of training, in our particular case students might not have been well informed on how to properly use a Verner Calliper for measurements. Additionally, it was noted that some callipers had zero error, thus measurements might not have been precise enough. Moreover, if the measured distance was too small (less than half a millimetre), a naked eye would not have been able to properly read the measurement easily. A good mitigation method would be to use a digital calliper for greater accuracy. Some additional errors that cannot have been reduced are: wearing of materials due to lab conditions and improper maintenance caused by sudden movements, and lack of machinery stability causing unwanted vibration when manufacturing is in process.

Manufacture Justification

Table 13: Decision matrix

| Criteria / Objectives | Effectiveness | Speed to solve | Feasibility | Impact on overall structure |
| --- | --- | --- | --- | --- |
| Sufficient flow rate | High-If sufficient flow rate is not supplied, peristaltic pump cannot work effectively | High – according modification of CAD Files | High-Greater force exerted by decreasing the distance of the wall casing aligned with the silicone tube and the bearings | High- with no flow rate all other components can not harmonically function producing the desired outcome |
| Spilling | High- If spilling occurs, peristaltic pump cannot be used and a waste of resources would occur (e.g., water) | Medium- modifying the component on CAD and rebuild adaptor with alternated, matching dimensions | Medium - hard to adapt the hose connector onto the mouth of the plastic bag/ lots of changes on CAD design’s dimensions need to be tested out | High-Liquid cannot flow through the pump; failure of functionality of the assembly |
| Rigidity | Medium- If the assemble is not rigid enough, it could still function but with lower effectiveness | Low- A lot of alternations need to be exerted on the whole assembly design on CAD | Medium- The whole assembly should be modified/ no control over the tolerance of the machinery | Medium- decrease in efficiency of waterflow rate/decreased accuracy and functionality |
| Pressure | High- without pressure exerted, no water flow can occur and thus no water can be moved | Medium- Modification on CAD file with according changes between the spacing of the wall and the bearings on the triangular spinner base | Medium- spacing between bearings and the wall need to be optimized to exert required force for continuous flow/done via tolerance analysis | High- undesired pressure can either cause collapse or decrease of functionality of the assembly |
| Size | Low – As long as the structure works efficiently, the size of the assemble doesn’t have a significant effect | Medium- Changes of dimensions performed on CAD files, ensuring uniform appearance of the assembly | High- a lot of alternations need to be done in the CAD files and the final assembly needs to be adapted accordingly/time consuming | Medium- defines the amount of water flowing through the pump |

Using the analysis made by the Decision matrix above, the most important key features that needed to be considered during the manufacturing process were: sufficient flow rate, spilling of liquid, rigidity of assembly, size and pressure exerted on the assembly. Classifying each category as high, medium, low, the assessment of effectiveness of these key components was completed. Concluding, the decision matrix stated that the most important key features were flow rate, spilling and pressure of the flow of water, resulting into their addressing throughout the design and prototyping process, while the rigidity and size were not considered as much

Key Design Characteristics

TRIANGULAR BASE

Firstly, a triangle shape was chosen to ensure efficient rotation and torque movement around the casing .3 bearings were attached onto the base to maximize contact time of the bearings with the silicon tube. The first parameter that was secured was the distance between two bearings. The more the bearings are separated, the greater the circumference of the rotation, leading to a larger amount of liquid pumped with one rotation. However, if the separation is too large, the system will not fit on the breadboard. With this into consideration, a distance of 30mm between the bearings was chosen. This would mean that the distance from one end of a pin to another had to be 66mm.

The second parameter that was secured was the radius of rotation. This had to be calculated in order to determine the diameter of the casing. Through Pythagoras, it was measured that the height from the base to the top of the centre pin was 58.5mm. To calculate the radius of the rotation of the pins, the height needed to be subtracted by the distance from the base to the centre of the middle bearing. It was found that the radius of the rotation without the bearings was 45.33mm and, after adding the distance of the outer section of the bearing, 52.33mm with the bearings on. This meant that the maximum diameter of the rotation of the centre piece was 105mm.

Taking the complexity of the triangular base into account, the 3D printer was used for its manufacture to ensure the fitting of bearings and cover slip were as accurate as possible.

Sketch 1: Triangular Base

U SHAPE LOWER AND UPPER CASINGS

The third parameter that was secured was the diameter of the casing. This needed to be found in order to ensure that the center piece can rotate freely whilst also deforming the tube enough to create a steady flow.  By adding the diameters of the tube to the diameter of the rotation, it is found that the maximum size of the casing is 119mm. However, to ensure that the tube deforms, the casing must be reduced in size. The value of the reduction of the diameter that was chosen was 4mm. This compresses the tube by ⅔ of the diameter whilst avoiding the risk of the printer producing a reduced print of the casing, meaning that the center piece would not be able to rotate. This concluded that the diameter of the casing would be 115mm. The casings were designed as large as possible to maximize the spacing used.

Sketch 2: U Shape lower Casing

It was also chosen that the casing would consist of 4 layers: 3 main pieces and a top layer. The top layer has a diameter of 105mm in order to cover the tube completely. The casing was chosen be made of 4mm layers instead of 3mm layers because the casing had to be taller that the height of the bearing pins (10mm). To achieve this, 4 layers would have to be printed for the 3mm layer and 3 layers for the 4mm. The more layers something has, the more they can slide over each other, thus the more they can move. Hence, the less layers the casing has, the less movement the casing has. This was essential since if the casing moved, the diameter of the casing could essentially increase, resulting in less deformation of the tube and thus less flow of liquid. With this, 3 layers of 4mm were chosen for the casing.  As the casing has a simple design, a laser cutter was used to speed up the manufacturing process and save time, by cutting pieces of acrylic with different dimensions as stated above (U Shaped).

TUBE CLAMP

Sketch 3: Tube Clamp

The fourth parameter that was secured was the size of the clamps. These constrained the movement of the tube and kept them in place at the entrance and exit of the casing. The bridge of the clamp had to have a height greater than 7mm, since the tube had a diameter of 7mm, but also small enough such that the tube cannot move around that much. It cannot be exactly 7mm as the printer has a chance of printing a reduced size of the model, meaning that the tube cannot fit in the clamp. It was chosen that 8mm would be the height and diameter of the bridge. Due to the thickness requirement of this component, 3D printing was used for the manufacturing process, allowing for the creation of wholes fitting the screws that connect it to the base.

ALLEN KEY SPINNER

Sketch 4: Allen Key Spinner

Next, the parameters for the Allen key holder were chosen. The hexagonal hole had to be centered directly above the midpoint of the rotation of the center piece, which was also the center of the middle bearing. The chosen width of the hole was 6.58mm since, much like with the height of the clamps, the hole had to be greater than the Allen key (6.4mm) but small enough to secure it inside. After this, the size of the pins was chosen. These fixed the holder to the center piece. Due to the moment of the Allen key, the pins would have to withstand a lot of force, therefore the diameter of the pins had to be relatively large. The chosen value for the diameter was 7.8mm. The next parameter secured was the distance between the pins. To calculate this, the diameter of the bearing (26mm) was added to a chosen value of 2mm spacing between the pins as well as the diameters of the pins. This resulted in a total distance of 45.6mm between the ends of the pins. Finally, the size of the body of the Allen key holder was chosen. The height of the body was 4mm since it was tall enough to withstand the force from the moment whist also reducing the amount of material consumed.  Due to the thickness and complex hole shape requirement of this component, 3D printing was used for the manufacturing process, allowing for the creation of wholes fitting the screws that connect it to the base.

PIPE ADAPTOR

The final parameters to be determined were for the tube adaptor. It was important to secure the parameters for this in order to ensure that the liquid flows from the bag into the tube. There was a risk that by trying to make the tube fit tighter on the tube adaptor, the printer could create a larger model which would result in the tube not fitting. Therefore, the adaptor

Sketch 4: Pipe Adaptor

needed diameters large enough to have a tight seal whilst being large enough for them to fit.

Since the inner diameter of the tube is 5mm, the outer diameter of the smaller section of the adaptor was also 5mm. The inner diameter of the mouth of the bag has a diameter of 8mm, therefore the outer diameter of the larger section of the diameter will also be 8mm. The thickness of the adaptor chosen was 1mm since it did not have to withstand much force and allowed the greatest volume of liquid to flow without the walls being too weak. The length of the adaptor did not require any specific calculations as long as each section was large enough for the according piece to fit without it slipping out. With this, the length of the larger section was 10mm and the smaller sections as 15mm. With the middle section transitioning from the larger to the smaller section having a length of 5mm, the total length of the adaptor was 30mm.

Design Constrains/Possible Mitigation Actions

Design Calculations and Prototyping

During the first stages of Data Analysis, the measurements on the 2 sample categories were taken and a mean value of those was calculated. However, a standard deviation value was calculated for all shapes separately. Thus, when it came to plotting, a single Gaussian graph was created using all the different SDs, resulting in an anomalous shape with several peaks and magnitudes.

Graph 1: Showing wrong tolerancing data manipulation

Instead, it was advised to categorize the shapes into groups (colour coded as shown in Tables 1&2), calculate the difference between the actual and average measurements and find an average value of SD, allowing the plot of different graphs of each shape. This resulted in symmetrical, normal distribution curves allowing tolerance of maximum and minimum values to be calculated.

CAD Difficulties

On the 3rd day of the scenario, the first prototype triangular spinner was successfully printed out, however, upon placing in upon the other assembly parts, there were some obvious issues that needed redesigning.

Our initial idea for making the motor is to have a hollow part in the middle of the triangle but this does not guarantee a perfect or suitable fit for the bearing to fit into in order to make the triangle motor spin. Another case to consider in the flaw of this design is that there is no platform or support to prevent the bearing and the pin holder from falling out mid-spin. We improved from this design a kind of hollowed-out cylindrical tunnel extrusion that extends to the same height of the breadboard on the triangle surface.

Figure 1 : Comparison of the existence of the cylindrical tunnel between old model{left} and latest model{right}

Our initial design also included the bearings to face upwards, having the middle bearing spinning when in contact with the base. However, this would potentially increase the height of the bearings, way above the breadboard where the silicone tube was attached; there was no contact between the bearings and the tube. This problem has been solved by placing the triangle and bearings to face downwards maximizing contact time and rotational movement of the triangular base

Figure 2 : The bearing orientation of the old model{left} and the latest model{right]

During the testing, it was observed that the bearings, when rotated together with the triangular base, did not extend enough to come in contact with the tube walls, resulting in a decreased force exerted on the tube when rotating; the flow of water was inefficient and it could be possible for some water to move backwards.

Figure 3 : The gap distance of the bearing and the casing of the old model and the latest model(right)

Figure 6: Real life image displaying the issue described above

As shown from the diagram above, there was a gap of 10.891mm between the casing and the bearings. With the outer diameter of the silicone tube being 7mm long, there will be an approximate spacing of 3mm between the bearings and the silicone tube which does not guarantee enough pressure was being exerted where required. The solution to this would be to increase the length of the sides of the triangular base, to compensate for the space in between the bearings and the silicone tube. After experimenting, a gap of distance of 2.452mm was achieved, providing a displacement/squeeze of the tube of approximately 5mm.

Additionally, an issue occurred when the first pipe adapter prototype was printed out, the mouth of the plastic bag was not able to fit into the opening of the hose connector, as the thickness of the opening diameter was too thick. As for the tube, it had to be stretched considerably before fitting into the smaller end opening. This may have caused wear and tear in the silicone tube after some time when in use. All measurements were taken in early stages, were based on the fact that the mouth of both the bag and the tube will fit inside the 3D hose connector model. To counter-act this issue, it was decided that the 3D print model will fit inside the mouth openings of both the bag and the silicon tube.

The prototype of the Allen keyhole, to allow spin of the triangular base, was miscalculated resulting in a larger diameter than the Allen key itself. So, the diameter was reduced to fit the Allen key into it.

Figure 5 : The diameters of the Allen key hole of the old model{right} and the latest model{right}

Another problem that we have dealt with was that the clamp of for the pipe was not linearly aligned with the entrance to the casing, as the clamp entrance was quite small and not symmetrical. A larger hole was introduced to allow more space for the silicone tube to be aligned and screwed on the base.

Figure 5: The change in hole size of the old model{right} and the latest model{left}

On the 5th day of the Scenario, there was an attempt to assemble the new larger clamp but the sides were obstructing the rotation pathway of the triangle spinner. The sides of the clamp had to be reduced to allow more leeway for the triangle spinner to move and rotate.

There was also a concern where the silicone tube may be displaced upwards sometime during the motion as there was no roof or cover to prevent it from happening. A top layer acrylic casing was made to overlap and act as a roof for the casing.

Figure 8 : Showing the issue of the silicone tube being displaced

CAD FILES OF PERISTALTIC PUMP

Figure ii: CAD of Tube Clamp

Figure i: CAD of Allen Key Spinner

Figure iii: CAD of Pump Adaptor

Figure iv: CAD of Pinhole

Figure v: CAD of U-Shaped Bottom Layer

Figure iv: CAD of U-Shaped Top Layer

Figure vii: CAD of Triangular Centre Piece

Figure viii: CAD of Fully Assembled Peristaltic Pump

ASSEMBLY

The first step in assembling the pump was to insert a nail through each hole of the stacked layers of the casing and the breadboard. Once this was done, the pinhole was inserted into the central bearing and placed into the middle-extruded hole in the triangular base. After pushing the rest of the bearings into their allocated extruded cylindrical pins on the triangular base, a nail was inserted into the pinhole. This nail would then be inserted into the hole on the breadboard in the centre of the casing semicircle (U-shape casing). On the bottom of the breadboard, a nut was screwed onto the nail to secure the middle bearing and thus fixed the triangular base to the breadboard.

For the tube, the mouth of the bag was fitted around the larger section of the pipe adaptor (back side) and the tube was fitted around the smaller cylindrical section. After this, the tube was placed onto the casing, having to compress the tube slightly where the tube and the bearings met. The holes of the top layer of the casing were placed such that the nails passed through them and a nut was screwed onto the nail. This secured the casing to the breadboard. Finally, the pins of the Allen key spinner were inserted into the holes on the triangular base. With this, the peristaltic pump was fully assembled, having the Allen key screwed into the top cover clip and rotating the structure around.

FEA Modelling

Simulation of Bearing deforming Tube

As the center piece of the pump rotates, the bearings roll over the tube. This creates a force onto the tube which causes it to deform. As a result, the fluid inside displaces in the direction of the rotation of the center piece, creating a flow within the system. This deformation can be both visualized and calculated via a simulation in Fusion 360.

To simulate this event, a simplified scenario with 3 individual components was created. These included: bearing, tube and base. Once they were all made to scale, their physical properties were added correspondingly. This was done by assigning each component a material type: Acrylic to the bearing, Rubber to the tube and Plastic to the base. Constraints were then added to restrict the components movement to their according axis. To ensure that the base acts like a base, it must be constrained on all axis, thus resulting in not being able to deform or move at all. The tube was then constrained to the x axis and the bearing to the x and y axis. This was done to make sure that the bearing only moved downwards into the tube and that the tube only deformed vertically and horizontally. After, contacts were created between the components to state the relationship the components have with each other. The tube and the base had a “bonded” contact as the tube would ideally follow the shape of the casing of the pump, and be in complete contact with the base at all times. The bearing and the tube had a “separation” contact which states that although they could permeate each other, they were not bonded. Finally, a vertical force was applied to the top of the bearing and a Linear Stress simulation could be created.

Figure 1: A simulation of the bearing moving downward on the tube with a force of 20N. The colors represent the stress at each point.

Figure 2: A simulation of the bearing moving downward on the tube with a force of 15N. The colors represent the stress at each point.

After various studies and simulations, the force found that causes the tube to completely fold is around 40N. Although knowing this helps in order to know the minimum force needed to flatten the tube, the real-life system will only have the tube deform to around 4mm in diameter. After experimenting with the simulation, it was found that a force of 15N was needed to achieve this.

Linear Static Stress Simulation

Figure 7: Stress distribution on rotating piece when applying torque on Allen key and reaction forces on the arms

Figure 8:  Strain on rotating piece when applying torque and reaction forces on the rotating piece

Figure _ and figure _ shows the stress and strain distribution on the rotating piece when force is applied on allen key. Assuming the force hand will provide of 2N on the Allen key. Constraint is applied on all the base of bearings and the bottom part of Allen key spinner as this position will move together when force is applied on the Allen key to rotate the whole structure. The bearing move together with the motion of Allen key spinner in a counter clockwise direction as there is a torque to it.

Figure 9: Stress and strain distribution on the rotating spinner when torque is applied on Allen key

Figure above have a similar stress and strain distribution from the previous figures. The key different is that the constraint is now concentrate on the pin below the Allen key. Providing different constraint will provide us the test to see the tight fit connections between a fixed spot and another component that is rotating which produces friction.

Figure 3: A side profile image of Figure 2.

Simulation of Force deforming Casing

Figure 4: A simulation of a force of 15N acting on the case. The colors represent the displacement at each point.

When the center piece rotated, it would not only apply a force onto the tube but also the casing. This could be once again visualized and simulated via Fusion 360. To do this, an accurately scaled 3D model of the case was created in a Non-Linear Stress simulation. Due to the library’s limited recourses, the material for the 3D printed casing needed to be created on Fusion. This was done by finding an existing and unused material and replacing its properties as well as name with that of PLA. As a result, the casing would have this material added to it. Constrains were then added to the holes on the casing. In order to simulate the nails inside of the holes, the inner face was constrained to all of the axis. Moreover, the base of the casing was constrained to the z axis as the whole system would only move on the x and y axis. A force perpendicular to the inner surface of the casing of 15N was then added and the simulation could be run.

As seen on Figure 4, the maximum displacement of the casing due to a force of 15N is 4.799 x 10-4 mm. This concludes that a force of 15N will not affect the shape of the casing, therefore the casing will never deform in the long term.

Figure 5: A simulation of the bearing moving downward on the tube with a force of 15N. The colors represent the strain at each point. The deformation has been exaggerated in order to help visualize the main areas of deformation.

Simulation of Bearing rotating around Pivot

As the bearing rolled on the tube, it rotated around a central pin. In order to visualize this, a Non-Linear Stress simulation was created with an accurately scaled bearing and a pillar. As there was no friction between the pillar and the bearing since it rotated around it self’s axis; no specific material is needed for either. Nevertheless, acrylic was added to the bearing. The pillar was constrained in all axis in order to simulate an actual pin and the bearing was constrained to the z axis in order to only allow movement on the x and y axis. A moment force was applied to the bearing of around 2Nm and a “separation

Figure 6: A simulation of the bearing rotating around a pin with a moment of 2Nm. The colors represent the displacement at each point.

contact” was created between both of them. Finally, the simulation could start.

Sustainability and Life cycle Analysis

LIFE CYCLE ANALYSIS

- Inventory

List of materials

- Polylactic Acid (PLA): triangle spinner, tube clamp,

- Acrylic: U shape

- Other materials:             -   Stainless steel (screws, bolts, part of bearings)

- PET (bearings)

PLA Material

Acrylic Material

Triangle Spinner / U-Shape Production

- Impact

For the PLA material, its production that involves the fermentation and polymerisation takes place at a high temperature. This process produces a large amount of carbon dioxide as its by-products alongside the main material. Carbon dioxide is a common greenhouse gas that contributes to global warming. The PLA material originates from renewable materials, corn and sugarcane. Though the manufacturing process produces large amount of carbon dioxide, the production of this material generates fewer greenhouse gases by 68% [1] (www.twi-global.com, n.d.). This is the upside of the material as it is relatively sustainable as the end-life scenario for this product consists of environmental-friendly options that includes recycling, composting and incineration. All the disposing options produces very minimal number of residues.

For the acrylic material, the main production process is polymerisation, which is similarly to the PLA, it produces carbon dioxide at a large amount. The difference between them is that the primary substance for acrylic is a chemical that is non-biodegradable, the methyl methacrylate. The overall process occurs at high temperature and involves toxic substance. Acrylic is impossible to recycle as it is non-biodegradable. The toxic substances in the material results in the very highly flammable properties of the material. Due to its nature, the usage of acrylic material is entirely unsustainable [2](How Products Are Made, 2014).

The production of the triangle spinner and U-shape casing is very simple and involves the direct use of material, PLA for 3D printing and acrylic sheets for laser cutting. A design process takes place beforehand and involves the usage of Fusion360 software that determines the dimensions for all the component involves. This process does not produce any large by product that is harmful. After the 3D printing and laser cutting, the parts will be assembled by hand to make up the whole pump component.

- Improvement

From the property’s description of PLA and acrylic, it is assumed that the usage of PLA as a prime material would be more sustainable to the environment as the analysis above proves that it is more biodegradable and effects the environment in less harmful way compared to the acrylic material.

To improve the whole sustainability of the peristaltic pump product, we could shift the material for the U-shape casing from acrylic to PLA. The method will now only involve the 3D printing. Though the overall time involve manufacturing lengthens, the long-term impact of this improvement to the environment is great as the overall carbon-emission to be produced is a lot less.

Figure Venn Diagram of the sustainability of the peristaltic pump we produced

Three components that defines the sustainability of a product are the environment, economy and society impact that it provides. Referring to the life cycle analysis, the main material of the pump, PLA is derived from renewable natural resources. The production process of this material emits less amount of greenhouse gases and use fewer energy which contributes in reducing carbon footprint.  This brings positive impact to the environment and proves that one of the key components of sustainability is reached.[3] (How Products Are Made, 2014).

From the economic perspective, this innovation of peristaltic pump will contribute in the economic growth as the functionality of this product caters to the demand of a certain group in the society. Moreover, this innovation is cost saving as the one of the production methods is less complex. For instance, some parts of this pump use laser cutting that is highly efficient and requires no man-operation during the process. It is timesaving because it could produce a large-scale amount of the product compartment at a short amount of time.

The society will benefit from this product through the healthcare sector as its functionality provide the ease for a treatment process. A standard of living of a person can improve entirely as there is the tool that contributes to their health progression. The low-cost production of this peristaltic pump makes this product accessible to everyone in the society. There is an equal opportunity for everyone in the society to benefit from this product in the healthcare sector. To conclude, this peristaltic pump provides benefits to the environment, economy and society overall. This proves that the product is sustainable.

Bill of Materials (BOM)

| BILLS OF MATERIALS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parts | Materials | Quantity | Price (Pounds) (x1) | Total price (Pounds) |
| Silicone Tube | Versilic@Silicone | 1 | 45.72 | 45.72 |
| Allen Hex Key | Chromium-Vanadium Steel | 1 | 0.75 | 0.75 |
| Genissy Liquid Bag | Stainless Steel | 2 | TBA | - |
| Radial Ball Bearing/26mm Outer diameter | SS316 Stainless Steel Balls Acetal/Polyoxymethylene cage | 4 | 1.36 | 5.44 |
| RS PRO Machine Screws | Stainless Steel | 7 | 10.26 | 71.82 |
| RS PRO Hex Nut | Bright Zinc Plated Steel | 7 | 29.22 | 204.54 |
| Farnell Vernier Callipers | Steel | 4 | 5.5 | 22 |
| PLA filament | - | 1 | 31.99 | 31.99 |
| Acrylic Sheet-4mm | - | 1 | 5 | 5 |
| Acrylic Breadboard | - | 1 | TBA | - |
| TOTAL |  | 29 | - | 387.26 |

Table 14: BOM

OFF-SHELF PARTS

| Part number | Part name | Qty in the design | Number of extra units needed in the 2nd pack | Notes |
| --- | --- | --- | --- | --- |
| 1 | Bearings | 4 | 0 | - excellent fatigue resistance - low moisture absorption - lightweight |
| 2 | Screws | 7 | 0 | - has a metric thread - made from high quality stainless steel - has strong resistance to abrasion and corrosion |
| 3 | Nuts | 7 | 0 | - hex type nut - accessible from a 360 degree angle due to shape - secure and tighten bolts faster and better |

Parts to Laser Print

| Assembly part number | Part name | Number of units needed | Any change from the standard settings for “Extruded acrylic” | Thickness of the acrylic sheet. Options are 3mm or 4mm | Name of the .dxf file submitted |
| --- | --- | --- | --- | --- | --- |
| 1 | Casing | 2 | None | 4 | FRIDAY FINAL U-SHAPED CASING v1.dxf |
| 2 | Toplayer Casing | 1 | None | 3 | FRIDAY FINAL TOPLAYER CASING v1/dxf |

Parts to 3D Print

| Part number | Part name | Qty Required (per product) | Any print settings you have changed from the “normal” settings | Estimated print time | Names of the .gcode and .stl file submitted |
| --- | --- | --- | --- | --- | --- |
| 1 | Pinhole | 1 | None | 5 minutes | FINAL FRIDAY PINHOLE v1.stl |
| 2 | Allen Keyhole | 1 | Thread Diameter=0.02mm | 26 minutes | FRIDAY FINAL ALLEN KEY SPINNER 4.0 v0 v1.stl |
| 3 | Pipe Adapter | 2 | None | 11 minutes | FRIDAY FINAL PIPE ADAPTER 6.0 v1.stl |
| 4 | Tube Clamp | 2 | Thread Diameter=0.02mm | 11 minutes | FRIDAY FINAL TUBE CLAMP v1.stl |
| 5 | Triangle Spinner | 1 | None | 1 hr 44 minutes | FRIDAY FINAL TRIANGLE WITH WALL 4.0 v1.stl |

APPENDIX

TAP DRAWINGS SCREENSHOTS

Figure A: TAP of Allen Key Spinner

Figure B: TAP of Pinhole

Figure C: TAP of U-Shaped Bottom Layer

Figure D: TAP of Tube Clamp

Figure E: TAP of Triangular Base

Figure F: TAP of Tube Adaptor

Figure G: TAP of U-Shaped Top Layer

REFERENCES

- www.twi-global.com. (n.d.). What is PLA? (Everything You Need To Know). [online] Available at: https://www.twi-global.com/technical-knowledge/faqs/what-is-pla#HowisitMade.

- How Products Are Made (2014). How acrylic plastic is made - material, making, used, processing, parts, Raw Materials. [online] Madehow.com. Available at: http://www.madehow.com/Volume-2/Acrylic-Plastic.html.

- www.twi-global.com. (n.d.). What is PLA? (Everything You Need To Know). [online] Available at: https://www.twi-global.com/technical-knowledge/faqs/what-is-pla#HowisitMade.

- 3DPrinting.com (2012). What is 3D printing? How does a 3D printer work? Learn 3D printing. [online] 3D Printing. Available at: https://3dprinting.com/what-is-3d-printing/.

- Monroe Engineering. (2020). The 3 Stages of 3D Printing and How They Work. [online] Available at: https://monroeengineering.com/blog/the-3-stages-of-3d-printing-and-how-they-work/.

- ‌Manufacturing GUIDE (ucl-eu-west-2-moodle-sitedata.s3.eu-west-2.amazonaws.com)

- https://ucl-eu-west-2-moodle-sitedata.s3.eu-west-2.amazonaws.com/86/f8/86f827ae313fe776c9db27ac3b7147bd4ee92ce1?response-content-disposition=inline%3B%20filename%3D%223D%20print%20Guide.pdf%22&response-content-type=application%2Fpdf&X-Amz-Content-Sha256=UNSIGNED-PAYLOAD&X-Amz-Algorithm=AWS4-HMAC-SHA256&X-Amz-Credential=AKIA47YHZF637GKGWUJC%2F20221102%2Feu-west-2%2Fs3%2Faws4_request&X-Amz-Date=20221102T222220Z&X-Amz-SignedHeaders=host&X-Amz-Expires=21580&X-Amz-Signature=f3b927d198818709fa94a18c70ce6922561c45dbb229211420706d17a97085a0